# 🎯 Presentation Guide: Single-Pass Drone Video to Accurate 3D Model Generation System

Problem Statement ID: SIH26158 | Smart India Hackathon Prototype

---

## 1. Executive Summary (The 30-Second Elevator Pitch)

"Our project solves SIH26158 by taking a single, unconstrained drone flyover video (not a pre-planned grid survey) and automatically converting it into an accurate, georeferenced 3D point cloud model.

>

The system automates video keyframe extraction, masks out moving dynamic objects (vehicles/humans), recovers shadow details using CLAHE contrast enhancement, ingests embedded GPS telemetry, and executes GPU-accelerated Structure-from-Motion (SfM) and Multi-View Stereo (MVS) via COLMAP on an RTX 3050 GPU, outputting an interactive 3D WebGL Dashboard with Architectural White Clay Mode, 3D Semantic Segmentation, and click-to-measure height analytics."

---

## 2. Key Challenges & Our Technical Solutions

| Key Problem Challenge (SIH26158) | Technical Solution Implemented |
| --- | --- |
| **(i) Single Flight Path (Limited Viewing Angles)** | **High-Overlap Sampling (6 FPS)**: Decimates video to ~100 keyframes with ~90% spatial overlap. |
| **(ii) Motion Blur & Compression Artifacts** | **Laplacian Variance Filtering**: Automatically rejects blurry keyframes scoring below sharpness threshold. |
| **(iii) Illumination & Shadows** | **CLAHE Contrast Recovery (`enhance_contrast.py`)**: Restores feature details in shadowed facades. |
| **(iv) Dynamic Objects (Vehicles/Humans)** | **Motion Object Masking (`mask_dynamic_objects.py`)**: Generates SIFT feature masks to filter moving traffic. |
| **(v & viii) GPS Inaccuracies & Metric Accuracy without GCPs** | **UTM Pose Priors & Georeferencing (`georeference_model.py`)**: Transforms WGS84 GPS to UTM cartesian pose priors. |
| **(vi) Processing Requirements** | **RTX 3050 CUDA Acceleration**: 16k SIFT features & 4K PatchMatch Stereo GPU execution. |

---

## 3. Desired Output & Evaluation Criteria Matrix

| Deliverable / Evaluation Criterion | Expected Outcome | System Result Achieved |
| --- | --- | --- |
| **(i) 3D Terrain & Structures** | Metrically accurate 3D model of building facades and roofs | **558,595 dense 3D points** & solid Octree Depth 12 Poisson mesh |
| **(ii) Building Height Analytics** | Calculate ground elevation & building height | **25.79 meters structure height** calculated automatically |
| **(iii) 3D Semantic Classification** | Classify buildings, roads, and vegetation | Multi-class 3D segmentation (Red: Buildings, Gray: Roads, Green: Trees) |
| **(iv) Georeferencing** | Real-world UTM metric coordinates | Georeferenced WGS84 UTM spatial placement |
| **(v) Interactive Visualization** | Multi-mode WebGL viewer with distance tools | **White Clay Mode**, Elevation Heatmap, RGB Color & Click-to-Measure Tool |

---

## 4. End-to-End System Architecture

┌────────────────────────────────────────────────────────┐

│             Raw Drone Video + SRT Telemetry            │

└───────────────────────────┬────────────────────────────┘

│

┌─────────────────────────┼─────────────────────────┐

│                         │                         │

┌────────┴────────┐       ┌────────┴────────┐       ┌────────┴────────┐

│  Smart 6 FPS    │       │ CLAHE Contrast  │       │ Dynamic Motion  │

│ Frame Sampling  │       │ Shadow Recovery │       │ Object Masking  │

└────────┬────────┘       └────────┬────────┘       └────────┬────────┘

│                         │                         │

└─────────────────────────┼─────────────────────────┘

│

┌──────────────┴──────────────┐

│  COLMAP RTX 3050 CUDA Engine│

│  16k SIFT + 4K PatchMatch   │

└──────────────┬──────────────┘

│

┌─────────────────────────┼─────────────────────────┐

│                         │                         │

┌────────┴────────┐       ┌────────┴────────┐       ┌────────┴────────┐

│  3D Height &    │       │ 3D Semantic     │       │ WebGL 3D        │

│ Metric Analytics│       │ Classification  │       │ Dashboard UI    │

└─────────────────┘       └─────────────────┘       └─────────────────┘

---

## 5. Potential Applications

1. Disaster Damage Assessment: Near real-time 3D building height & collapse analysis.

2. Military Reconnaissance & Border Mapping: Single-pass flyover terrain mapping without pre-planned grid surveys.

3. Urban Planning & Digital Twins: High-accuracy 3D city models with automatic height extraction.

4. Infrastructure Inspection: Rapid inspection of bridges, highways, and high-rise structures.